| Preparing of tests 1.Rating of appliance |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| V | Hz | W | A |  | Class | IP |
| Climate type: |  |  |  |  |  |  |
| 2.Classification of appliance ☐ hand-held appliance ☐ portable appliance ☐ stationary appliance ☐ fixed appliance ☐ build-in appliance ☐ appliance for unattended use ☐ appliance for attended use ☐ tested as motor-operated appliance ☐ tested as heating appliance ☐ tested as combined appliance ☐ with fixed power cord type attachment ☐ with appliance inlet ☐ permanently connected to fixed wiring ☐ with pin ☐ appliance with electronic circuits,circuit diagram with abnomal conditions attached. ☐other important information: . 3. General conditions for the tests For clauses 10, 11, 13 and subclause 19.103: Ambient temperature: ☐32±1°C ☐38±1°C ☐43±1°C ☐23±2°C ☐ other: . For other tests: Ambient temperature: ☐20±5°C ☐ other: . Special calculation: ☐Appliance with rated voltage range and rated power input: ☐Appliance with PTC heating element: ☐Other: . ☐Special order of tests : . ☐Controls or devices adjusted to their most unfavourable setting |  |  |  |  |  |  |
| 4. Selection for the tests |  |  |  |  |  |  |
| ☐Power input and current ☐ leakage current and electric strength at OT ☐ Moisture resistance test ☐ Overload protection of transformer and associated circuits ☐ Stability and mechanical hazards ☐ Construction ☐ Components ☐ Terminals for external conductors ☐ Torque test for screws and nuts ☐Resistance to rusting ☐Annex H: switches ☐Annex T: UV-C radiation effect on non-metallic materials |  |  |  | ☐ Heating test ☐Transient overvoltages ☐ leakage current and electric strength ☐ Endurance test ☐ Abnormal operation test ☐ Mechanical strength test ☐ Internal wiring ☐ supply connections and external flexible cords ☐ provision for earthing ☐ Resistance to heat and fire ☐Annex D: thermal motor protectors ☐Annex G: safety isolating transformer ☐Annex N: Proof tracking test ☐Annex EE Test for material encasing and in contact with thermal insulation |  |  |

Test Location: ☐1/F., Building 11, Scholar Innovation Park, No.1188, Zhongguan Road, Zhenhai District, Ningbo, Zhejiang, China ☐3F Building C13, R&D Park, No.32, Lane 299 Guanghua Road, National Hi-Tech Zone, Ningbo, 315048, P. R. China

Filled by Test Engineer Attachment(s): ☐ Equipment List attached in test plan. total ____ page(s) ☐ Temperature record (curve) ☐ Others:________

| Testing starting date: |  | Testing finish date: |  |
| --- | --- | --- | --- |
| Tested by: |  | Reviewed by: |  |

| ☐7.14 | RUBBING TEST FOR RATING LABEL |  |  |
| --- | --- | --- | --- |
| Test date: |  | Ambient: |  |
| Equipment ID: |  | Sample ID |  |
| Equipment No: |  |  |  |
| -- | The rating label was checked by rubbing the marking for 15s with a piece of cotton cloth soaked with water and again for 15s with petroleum. Is the label curled? ☐Yes, ☐No Is it easily to remove marking plates? ☐Yes, ☐No Is the marking legible? ☐Yes, ☐No Result: ☐PASS ☐FAIL |  |  |

| 8.1.4& 22.42 | Test for protective impedance |  |  |  |
| --- | --- | --- | --- | --- |
| Test date: |  | Ambient: |  |  |
| Equipment ID: |  | Sample ID |  |  |
|  | Test condition: the appliance being supplied at rated voltage If protective impedance is used, the current between the part and the supply source shall not exceed 2 mA for d.c., its peak value shall not exceed 0,7 mA for a.c. and – for voltages having a peak value over 42,4 V up to and including 450 V, the capacitance shall not exceed 0,1 uF; – for voltages having a peak value over 450 V up to and including 15 kV, the discharge shall not exceed 45 uC; – for voltages having a peak value over 15 kV, the energy in the discharge shall not exceed 350 mJ. If any one of the protective impedance components is short-circuited or open-circuited, the values specified in 8.1.4 shall not be exceeded. |  |  |  |
| Result: | Input Test 1: 1): Between output: The voltage with no load is ; The voltage with E load is ; 2): Between Transformer output: The voltage with no load is ; The voltage with E load is ; Limit: 42.4V |  |  |  |
|  | Input Test 2: The current between the part and the supply source: mA (limit: 0.7 mA); With CY2 or CY1 short-circuit: The current between the part and the supply source: mA (limit: 0.7 mA); With CY2 or CY1 open-circuit: The current between the part and the supply source: mA (limit: 0.7 mA); |  |  |  |
|  | Test 3: The peak value of voltage is V; The capacitance is uF or the discharge is uC. |  |  |  |

| ☐ 10.1 Power input measurement |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test date: |  |  | Ambient: |  |  |  |
| Equipment ID: |  |  | Sample ID |  |  |  |
| Definition of normal operation: ☐Normal operation of a refrigerating appliance: Operation at an ambient temperature in accordance with 5.7, empty, with the doors and lids closed. User-adjustable temperature control devices which control the operation of the motor-compressor in compression-type appliances, are short-circuited or otherwise rendered inoperative. ☐Normal operation of an ice-marker: Operation at an ambient temperature in accordance with 5.7, with the supply water at a temperature of 15ºC±2ºC. ☐Normal operation of an incorporated ice-marker: Operation at the normal temperature of the frozen food storage compartment, with the supply water at a temperature of 15ºC±2ºC. ☐Normal operation of an ice-cream appliance: Operation of the appliance using the maximum quantity of the mixture of ingredients indicated in the instructions; the mixture used being that which gives the most unfavourable results, the mixture being at an initial temperature of 23ºC±2ºC |  |  |  |  |  |  |
| Input deviation of/at: |  | P rated (W) |  | P measured (W) |  | dP |
| ☐ 10.2 Current measurement |  |  |  |  |  |  |
| Test date: |  |  | Ambient: |  |  |  |
| Equipment ID: |  |  | Sample ID |  |  |  |
| Current deviation of/at: |  | I rated (A) |  | I measured (A) |  | dI |
| ☐ 10.101, 10.102 Heating system |  |  |  |  |  |  |
| Test date: |  |  | Ambient: |  |  |  |
| Equipment ID: |  |  | Sample ID |  |  |  |
| Current deviation of/at: |  | I rated (A) |  | I measured (A) |  | dI |

| ☐ 11.Heating test |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test date: |  |  |  |  |  |  |  |  |  |  |  | Ambient: |  |  |  |  |  |  |  |
| Equipment ID: |  |  |  |  |  |  |  |  |  |  |  | Sample ID |  |  |  |  |  |  |  |
| Test location |  | ☐ Build-in appliance: installed in accordance with the instructions for installaiton. ☐ Ice-cream appliance: placed as near to the walls of the test corner as possible. ☐ other appliance: placed in a test enclosure. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test voltage |  | ☐ Heating appliances: corresponding to: W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | ☐ Motor-operated and combined appliances: V and V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test time |  | ☐ until steady conditions established. ☐ other: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Thermostat |  | ☐ set at the lowest temperature ☐ short-circuited ☐ without thermostat |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test voltage(V) |  |  |  |  |  |  | Current(A) |  |  |  |  |  |  | Power input(W) |  |  |  |  |  |
| Test voltage(V) |  |  |  |  |  |  | Current(A) |  |  |  |  |  |  | Power input(W) |  |  |  |  |  |
| No. | Position |  |  |  |  |  |  |  | At V |  |  |  |  | At V |  |  |  |  | Limit T(K) |
|  |  |  |  |  |  |  |  |  | t (C) |  |  |  | T(K) |  | t (C) |  |  | T(K) |  |
|  | (please specify in part 2 if possible) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (may be two or more pages) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Resistance method |  |  |  | ambient t1 |  |  |  |  |  |  | ambient t2 |  |  |  |  |  |  |  |  |
| Temperature rise of winding |  |  |  |  |  | R1(Ω) |  |  |  |  | R2(Ω) |  |  |  |  |  | Temperature rise(K) |  |  |
| Check: ☐ Protective devices other than self-resetting thermal motor-protectors for motor compressors shall not operate ☐when steady conditions established, self-resetting thermal motor-protectors for motor compressors shall not operate ☐ Sealing compound did not flow out |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ☐ 11.101 Heating test |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test date: |  |  |  |  |  |  |  |  |  |  |  | Ambient: |  |  |  |  |  |  |  |  |
| Equipment ID: |  |  |  |  |  |  |  |  |  |  |  | Sample ID |  |  |  |  |  |  |  |  |
| Test procedure |  | If the temperature of winding of motor compressors other than those complying with IEC 60335-2-34 including its Annex AA are higher than the temperature limit _____. The test is carried out again. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test location |  | ☐ Build-in appliance: installed in accordance with the instructions for installaiton. ☐ Ice-cream appliance: placed as near to the walls of the test corner as possible. ☐ other appliance: placed in a test enclosure. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test voltage |  | ☐ Heating appliances: corresponding to: W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | ☐ Motor-operated and combined appliances: V and V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test time |  | ☐ until steady conditions established. ☐ other: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Thermostat |  | ☐ set at the lowest temperature ☐ short-circuited ☐ without thermostat |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test voltage(V) |  |  |  |  |  |  | Current(A) |  |  |  |  |  |  |  | Power input(W) |  |  |  |  |  |
| Test voltage(V) |  |  |  |  |  |  | Current(A) |  |  |  |  |  |  |  | Power input(W) |  |  |  |  |  |
| No. | Position |  |  |  |  |  |  |  | At V |  |  |  |  |  |  | At V |  |  |  | Limit T(K) |
|  |  |  |  |  |  |  |  |  | t (C) |  |  |  |  | T(K) |  | t (C) |  |  | T(K) |  |
|  | (please specify in part 2 if possible) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (may be two or more pages) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Resistance method |  |  |  | ambient t1 |  |  |  |  |  |  | ambient t2 |  |  |  |  |  |  |  |  |  |
| Temperature rise of winding |  |  |  |  |  | R1(Ω) |  |  |  |  | R2(Ω) |  |  |  |  |  |  | Temperature rise(K) |  |  |
| Check:☐ Protective devices did not operate ☐ Sealing compound did not flow out |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ☐ 11.102 Heating test (defrosting system) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test date: |  |  |  |  |  |  |  |  |  |  |  | Ambient: |  |  |  |  |  |  |  |
| Equipment ID: |  |  |  |  |  |  |  |  |  |  |  | Sample ID |  |  |  |  |  |  |  |
| Test procedure |  | The appliance is supplied at the most unfavourable voltage between 0.94 and 1.06 times the rated voltage until the evaporator is coated with a layer of frost. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test location |  | ☐ Build-in appliance: installed in accordance with the instructions for installaiton. ☐ Ice-cream appliance: placed as near to the walls of the test corner as possible. ☐ other appliance: placed in a test enclosure. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test voltage |  | ☐ for absorption-type appliances and for compression-type appliances in which the defrosting system can be energized with the rest of the appliance unenergized: _________W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | ☐ for other compression-type appliances: V and V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test time |  | ☐ until steady conditions established. ☐ other: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Thermostat |  | ☐ set at the lowest temperature ☐ short-circuited ☐ without thermostat |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test voltage(V) |  |  |  |  |  |  | Current(A) |  |  |  |  |  |  | Power input(W) |  |  |  |  |  |
| Test voltage(V) |  |  |  |  |  |  | Current(A) |  |  |  |  |  |  | Power input(W) |  |  |  |  |  |
| No. | Position |  |  |  |  |  |  |  | At V |  |  |  |  |  | At V |  |  |  | Limit T(K) |
|  |  |  |  |  |  |  |  |  | t (C) |  |  |  | T(K) |  | t (C) |  |  | T(K) |  |
|  | (please specify in part 2 if possible) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (may be two or more pages) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Resistance method |  |  |  | ambient t1 |  |  |  |  |  |  | ambient t2 |  |  |  |  |  |  |  |  |
| Temperature rise of winding |  |  |  |  |  | R1(Ω) |  |  |  |  | R2(Ω) |  |  |  |  |  | Temperature rise(K) |  |  |
| Check:☐ Protective devices did not operate ☐ Sealing compound did not flow out |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ☐ 11.103 Heating test (heating system, other than defrosting systems) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test date: |  |  |  |  |  |  |  |  |  |  |  | Ambient: |  |  |  |  |  |  |  |  |
| Equipment ID: |  |  |  |  |  |  |  |  |  |  |  | Sample ID |  |  |  |  |  |  |  |  |
| Test location |  | ☐ Build-in appliance: installed in accordance with the instructions for installaiton. ☐ Ice-cream appliance: placed as near to the walls of the test corner as possible. ☐ other appliance: placed in a test enclosure. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test voltage |  | ☐ for absorption-type appliances and for compression-type appliances in which the defrosting system can be energized with the rest of the appliance unenergized: _________W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | ☐ for other compression-type appliances: V and V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test time |  | ☐ until steady conditions established. ☐ other: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Thermostat |  | ☐ set at the lowest temperature ☐ short-circuited ☐ without thermostat |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test voltage(V) |  |  |  |  |  |  | Current(A) |  |  |  |  |  |  | Power input(W) |  |  |  |  |  |  |
| Test voltage(V) |  |  |  |  |  |  | Current(A) |  |  |  |  |  |  | Power input(W) |  |  |  |  |  |  |
| No. | Position |  |  |  |  |  |  |  | At V |  |  |  |  |  | At V |  |  |  |  | Limit T(K) |
|  |  |  |  |  |  |  |  |  | t (C) |  |  |  | T(K) |  | t (C) |  |  | T(K) |  |  |
|  | (please specify in part 2 if possible) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (may be two or more pages) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Resistance method |  |  |  | ambient t1 |  |  |  |  |  |  | ambient t2 |  |  |  |  |  |  |  |  |  |
| Temperature rise of winding |  |  |  |  |  | R1(Ω) |  |  |  |  | R2(Ω) |  |  |  |  |  | Temperature rise(K) |  |  |  |
| Check:☐ Protective devices did not operate ☐ Sealing compound did not flow out |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ☐ 13.2 Leakage current at operation temperature |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test date: |  |  |  | Ambient: |  |  |  |
| Equipment ID: |  |  |  | Sample ID |  |  |  |
| Heating appliances: 1.15 x rated input : |  |  |  |  |  |  |  |
| Motor-operated and combined appliances: 1.06 x rated voltage : |  |  |  |  |  |  |  |
| Leakage current between |  |  | I (mA) |  |  | Max. allowed I (mA) |  |
| ☐ 13.3 Electric strength at operation temperature |  |  |  |  |  |  |  |
| Test date: |  |  |  | Ambient: |  |  |  |
| Equipment ID: |  |  |  | Sample ID |  |  |  |
| Test voltage applied between: |  |  | Voltage (V) |  |  | Breakdown (Yes/No) |  |
| ☐ 13.3,16.3, 24.5 Working voltage measuring |  |  |  |  |  |  |  |
| Test date: |  |  |  | Ambient: |  |  |  |
| Equipment ID: |  |  |  | Sample ID |  |  |  |
| Voltage supplied(V) |  | Test points |  |  | Working voltage(V) |  |  |

| Preparation: Detachable parts are removed. ☐ 15.1 IP test |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test date: |  |  |  |  |  |  | Ambient: |  |  |  |  |  |  |
| Equipment ID: |  |  |  |  |  |  | Sample ID |  |  |  |  |  |  |
| Degree of protection against water |  |  |  |  |  |  |  |  | Test time |  |  |  |  |
| Classification of installation and use |  |  |  | ☐Hand-held appliance are turned continuously |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ☐Build-in appliance are installed according to the instruction |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ☐Appliances used on floor or table are placed on a support |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ☐Other appliances |  |  |  |  |  |  |  |  |  |
| Check test result |  |  |  | ☐There is water on live parts |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ☐There is water on basic insulation of internal wires |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ☐There is a trace of water which reduce cr/cr below the requrements of clause 29 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ☐There is no trace of hazard water. |  |  |  |  |  |  |  |  |  |
| ☐ 15.2 spillage test |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test date: |  |  |  |  |  |  | Ambient: |  |  |  |  |  |  |
| Equipment ID: |  |  |  |  |  |  | Sample ID |  |  |  |  |  |  |
| Rated capacity of appliance(l) |  |  |  |  |  | Additional amount of water(l) |  |  |  |  |  |  |  |
| Check test result |  |  |  | ☐There is water on live parts |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ☐There is water on basic insulation of internal wires |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ☐There is a trace of water which reduce cr/cr below the requrements of clause 29 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ☐There is no trace of hazard water. |  |  |  |  |  |  |  |  |  |
| ☐ 15.101.1 spillage test |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test date: |  |  |  |  |  |  | Ambient: |  |  |  |  |  |  |
| Equipment ID: |  |  |  |  |  |  | Sample ID |  |  |  |  |  |  |
| Test procedure | The apparatus shown in figure 101 is filled with water containing 1% NaCl and 0.6% of acid rinsing agent. The apparatus is supported with its base horizontal and placed at the most unfavorable position. This test may be repeated as many times as necessary in different positions. |  |  |  |  |  |  |  |  |  |  |  |  |
| Check test result |  |  |  | ☐ withstand the test of 16.3 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ☐There is water on live parts |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ☐There is a trace of water which reduce cr/cr below the requrements of clause 29 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ☐There is no trace of hazard water. |  |  |  |  |  |  |  |  |  |
| ☐ 15.101.2 spillage test |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test date: |  |  |  |  |  |  | Ambient: |  |  |  |  |  |  |
| Equipment ID: |  |  |  |  |  |  | Sample ID |  |  |  |  |  |  |
| Test procedure |  | A rectangular container 200 mm x 110 mm and a height of 50 mm is filled with 0,5 l of the spillage solution. |  |  |  |  |  |  |  |  |  |  |  |
|  |  | The container is positioned, with its longest side parallel to the wall to be tested, on the highest shelf on which it will fit, the shelf shall have a clearance to the ceiling of the compartment of at least 130 mm. The appliance is disconnected from the supply and the solution in the vessel is discharged over the back and side interior walls of the cabinet or compartment including any electrical components mounted thereon, in the most unfavourable manner within a period of 2 s. |  |  |  |  |  |  |  |  |  |  |  |
|  |  | the test may be repeated as many times as necessary in different positions |  |  |  |  |  |  |  |  |  |  |  |
| Check test result |  |  |  | ☐ withstand the test of 16.3 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ☐There is water on live parts |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ☐There is a trace of water which reduce cr/cr below the requrements of clause 29 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ☐There is no trace of hazard water. |  |  |  |  |  |  |  |  |  |
| ☐ 15.103 spillage test |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test date: |  |  |  |  |  |  | Ambient: |  |  |  |  |  |  |
| Equipment ID: |  |  |  |  |  |  | Sample ID |  |  |  |  |  |  |
| Test procedure | Appliance, other than built-in appliance, ice-markers and ice-cream appliances are titled at an angle of up to 2º in relation to the position of normal use in the direction which is likely to be the most unfavourable for this test. One half-litre of water containing 1% NaCl and 0.6% of acid rinsing agent, is poured uniformly over the top of appliance in approximately 60s at the most unfavourable place from a height 50mm with the controls in the on position and the appliance disconnected from the supply. |  |  |  |  |  |  |  |  |  |  |  |  |
| Check test result |  |  |  | ☐ withstand the test of 16.3 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ☐There is water on live parts |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ☐There is a trace of water which reduce cr/cr below the requrements of clause 29 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ☐There is no trace of hazard water. |  |  |  |  |  |  |  |  |  |
| ☐ 15.104 spillage test |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test date: |  |  |  |  |  |  | Ambient: |  |  |  |  |  |  |
| Equipment ID: |  |  |  |  |  |  | Sample ID |  |  |  |  |  |  |
| Test procedure | For ice-markers that are directly connected to the water supply, the container is filled with water as in normal use. The inlet valve is held open and filling is continued for 1min after the first evidence of overflow. Where no spillage occurs due to operation of a device that prevents such spillage, the inlet valve is held open for further 5min following the operation of this device. |  |  |  |  |  |  |  |  |  |  |  |  |
| Check test result |  |  |  | ☐ withstand the test of 16.3 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ☐There is water on live parts |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ☐There is a trace of water which reduce cr/cr below the requrements of clause 29 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ☐There is no trace of hazard water. |  |  |  |  |  |  |  |  |  |
| ☐ 15.105 spillage test |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test date: |  |  |  |  |  |  | Ambient: |  |  |  |  |  |  |
| Equipment ID: |  |  |  |  |  |  | Sample ID |  |  |  |  |  |  |
| Test procedure | Immediately after the test of 11.102 |  |  |  |  |  |  |  |  |  |  |  |  |
| Check test result |  |  |  | ☐ withstand the test of 16.3 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ☐There is water on live parts |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ☐There is a trace of water which reduce cr/cr below the requrements of clause 29 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ☐There is no trace of hazard water. |  |  |  |  |  |  |  |  |  |
| ☐ 15.3 Humidity test |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test date: |  |  |  |  |  |  | Ambient: |  |  |  |  |  |  |
| Equipment ID: |  |  |  |  |  |  | Sample ID |  |  |  |  |  |  |
| Test temperature(°C) |  |  |  |  |  |  |  | Humidity(%) |  |  |  |  |  |
| Check test result by tests of clause 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ☐ 16.2 Leakage current |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test date: |  |  |  |  |  |  | Ambient: |  |  |  |  |  |  |
| Equipment ID: |  |  |  |  |  |  | Sample ID |  |  |  |  |  |  |
| Single phase appliances: 1.06 x rated voltage(V): |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Three phase appliances 1.06 x rated voltage divided by 3(V) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Leakage current between |  |  |  |  | I (mA) |  |  |  |  | Max. allowed I (mA) |  |  |  |
| ☐ 16.3 Electric strength |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test date: |  |  |  |  |  |  | Ambient: |  |  |  |  |  |  |
| Equipment ID: |  |  |  |  |  |  | Sample ID |  |  |  |  |  |  |
| Test voltage applied between: |  |  |  |  | Voltage (V) |  |  |  |  | Breakdown (Yes/No) |  |  |  |
| Remark: |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ☒ 17. Overload protection of transformer and associated circuits |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test date: |  |  |  |  |  |  |  |  |  |  |  | Ambient: |  |  |  |  |  |  |  |
| Equipment ID: |  |  |  |  |  |  |  |  |  |  |  | Sample ID |  |  |  |  |  |  |  |
| Type of sample |  |  |  |  |  |  |  |  |  |  | Tf of thermal cut-out(°C) |  |  |  |  |  |  |  |  |
| Test sample 1 |  | operating time of thermal cut out: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test voltage(V) |  |  |  |  |  | Current(A) |  |  |  |  |  |  |  |  | Power input(W) |  |  |  |  |
| No. | Position |  |  |  |  |  |  | t (C) |  |  |  |  | T(K) |  |  |  |  | Limit T(K) |  |
| Winding temperature |  |  |  | t1(°C) |  |  | R1(Ω) |  | t2(°C) |  |  |  |  | R2(Ω) |  | Temperature (°C) |  |  |  |
| Test sample 2 |  |  | operating time of thermal cut out: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test voltage(V) |  |  |  |  |  | Current(A) |  |  |  |  |  |  |  |  | Power input(W) |  |  |  |  |
| No. | Position |  |  |  |  |  |  | t (C) |  |  |  |  | T(K) |  |  |  |  | Limit T(K) |  |
| Winding temperature |  |  |  | t1(°C) |  |  | R1(Ω) |  | t2(°C) |  |  |  |  | R2(Ω) |  | Temperature (°C) |  |  |  |

| ☐ 17. Overload protection of transformer and associated circuits for fail-safe transformers |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test date: |  |  |  |  |  |  |  |  |  | Ambient: |  |  |  |  |  |  |  |
| Equipment ID: |  |  |  |  |  |  |  |  |  | Sample ID |  |  |  |  |  |  |  |
| See EN 61558-1 clause 15.5: Test 1:Eath of the three specimens is mounted as in normal use on a 20mm thick dull black painted plywood surface and heated in accordance with 14.2 until the temperature stabilize and then operated at 1,1 times the rated input voltage, the output winding which produced the highest temperature during the test of 14.2 being initially loaded with 1,5 times the rated output current until steady-state conditions are reached or the transformer fails Test 2:If the transformer does not fail, the time to reach steady-state conditions is noted, and the chosen output winding is then short-circuited. The test is continued until the transformer fails. Each specimen shall fail within the time duration noted and not exceeding 5h |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type of sample |  |  |  |  |  |  |  |  | Tf of thermal link(°C) |  |  |  |  |  |  |  |  |
| Test sample 1 |  |  | Test voltage(V) |  |  | Input Current(A) |  | Power input(W) |  |  |  |  | Output current(A) |  |  |  | Remark |
| No. | Position |  |  |  |  |  | t (C) |  |  |  | T(K) |  |  |  |  | Limit T(K) |  |
| Test sample 2 |  |  | Test voltage(V) |  |  | Input Current(A) |  | Power input(W) |  |  |  |  | Output current(A) |  |  |  | Remark |
| No. | Position |  |  |  |  |  | t (C) |  |  |  | T(K) |  |  |  |  | Limit T(K) |  |
| Test sample 3 |  |  | Test voltage(V) |  |  | Input Current(A) |  | Power input(W) |  |  |  |  | Output current(A) |  |  |  | Remark |
| No. | Position |  |  |  |  |  | t (C) |  |  |  | T(K) |  |  |  |  | Limit T(K) |  |
| Electric strength |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test voltage applied between: |  |  |  |  | Voltage (V) |  |  |  |  |  |  | Breakdown (Yes/No) |  |  |  |  |  |

| ☐ 19.1 Abnormal operation (Annex AA for fan motors) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test date: |  |  |  |  |  |  |  |  | Ambient: |  |  |  |  |  |  |
| Equipment ID: |  |  |  |  |  |  |  |  | Sample ID |  |  |  |  |  |  |
| The fan and its motor are mounted on wood or similar material. The motor rotor is locked. Fan blades and motor brackets are not removed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test voltage(V) |  |  |  |  |  | Current(A) |  |  |  |  | Power input(W) |  |  |  |  |
| Test time |  | ☐ until steady conditions established. ☐ other: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| No. | Position |  |  |  |  |  | t (C) |  |  | T(K) |  |  |  | Limit T(K) |  |
| Check: ☐ self-resetting thermal cut-out ☐ non-self-resetting thermal cut-out ☐ thermal link ☐ No thermal cut-out operated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Operating time of thermal motor-protector (min) |  |  |  |  |  |  |  | Operating temperature of thermal motor-protector (°C) |  |  |  |  |  |  |  |
| ☐ 72hours after the beginning of test, the motor shall withstand the test of clause 16.3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test voltage applied between: |  |  |  |  |  |  | Voltage (V) |  |  |  |  | Breakdown (Yes/No) |  |  |  |
| ☐ At the end of test, the leakage current is measured between the windings and the body at a voltage equal to twice the rated voltage. The value shall not exceed 2mA. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ☐ The appliance emitted flames, molten metal, poisonous or ignitable gas in hazardous amounts ☐ Enclosures had deformed to such an extent that compliance with cl. 8 is impaired ☐ Appliance can still be operated and it didn’t comply with 20.2 ☐ the appliance shall not undergo a dangerous malfunction ☐ there shall be no failure of protective electronic circuits if the appliance is still operable ☐ No above damage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ☐ 19.4 Abnormal operation |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test date: |  |  |  |  |  |  |  | Ambient: |  |  |  |  |  |
| Equipment ID: |  |  |  |  |  |  |  | Sample ID |  |  |  |  |  |
| Appliances are tested under conditions specified in Cl. 11 but thermal control(s) operated during the test of Cl. 11 is(are) short-circuited(in turn). |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test voltage(V) |  |  |  |  | Current(A) |  |  |  |  | Power input(W) |  |  |  |
| No. | Position |  |  |  |  | t (C) |  |  | T(K) |  |  | Limit T(K) |  |
| Check: ☐ self-resetting thermal cut-out ☐ non-self-resetting thermal cut-out ☐ thermal link ☐ No thermal cut-out operated |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Operating time of thermal cut out(min) |  |  |  |  |  |  | Operating temperature of thermal cut out(°C) |  |  |  |  |  |  |
| ☐ The appliance emitted flames, molten metal, poisonous or ignitable gas in hazardous amounts ☐ Enclosures had deformed to such an extent that compliance with cl. 8 is impaired ☐ Appliance can still be operated and it didn’t comply with 20.2 ☐ the appliance shall not undergo a dangerous malfunction ☐ there shall be no failure of protective electronic circuits if the appliance is still operable ☐ The temprature of the housing of motor-compressors other than those which comply with IEC 60335-2-34 shall not exceed 150ºC. ☐ No above damage |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ☐ 19.6 Abnormal operation |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test date: |  |  |  |  |  |  |  | Ambient: |  |  |  |  |  |
| Equipment ID: |  |  |  |  |  |  |  | Sample ID |  |  |  |  |  |
| Appliances with PTC heating elements tested at rated voltage, establishing steady conditions The working voltage of the PTC heating element is increased by 5% and the appliance is operated until steady conditions are re-established. The voltage is then increased in similar steps until 1.5 times working voltage or until the PTC heating element ruptures. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test voltage(V) |  |  |  |  | Current(A) |  |  |  |  | Power input(W) |  |  |  |
| No. | Position |  |  |  |  | t (C) |  |  | T(K) |  |  | Limit T(K) |  |
| Check: ☐ self-resetting thermal cut-out ☐ non-self-resetting thermal cut-out ☐ thermal link ☐ No thermal cut-out operated |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Operating time of thermal cut out(min) |  |  |  |  |  |  | Operating temperature of thermal cut out(°C) |  |  |  |  |  |  |
| ☐ The appliance emitted flames, molten metal, poisonous or ignitable gas in hazardous amounts ☐ Enclosures had deformed to such an extent that compliance with cl. 8 is impaired ☐ Appliance can still be operated and it didn’t comply with 20.2 ☐ the appliance shall not undergo a dangerous malfunction ☐ there shall be no failure of protective electronic circuits if the appliance is still operable ☐ The temprature of the housing of motor-compressors other than those which comply with IEC 60335-2-34 shall not exceed 150ºC. ☐ No above damage |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ☐ 19.7 Locking the motor |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test date: |  |  |  |  |  |  |  |  |  |  |  |  | Ambient: |  |  |  |  |  |  |  |  |  |  |
| Equipment ID: |  |  |  |  |  |  |  |  |  |  |  |  | Sample ID |  |  |  |  |  |  |  |  |  |  |
| Type of motor |  |  |  |  |  |  |  |  |  |  |  | Class of winding |  |  |  |  |  |  |  |  |  |  |  |
| Test time |  | ☐ 30s ☐ the maximum period allowed by the timer or programmer ☐ 5min ☐ until steady conditons are established ☐ 5min for fan motors of ice-cream appliance. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Locking means |  | ☐ capacitors in the circuit of an auxiliary winding, being open-circuited one at a time. ☐ test repeated with capacitors short-circuited one at a time unless capacitor of P2 ☐ Locking rotor or locking moving parts |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test voltage(V) |  |  |  |  |  |  | Current(A) |  |  |  |  |  |  |  |  | Power input(W) |  |  |  |  |  |  |  |
| No. | Position |  |  |  |  |  |  |  | t (C) |  |  |  |  | T(K) |  |  |  |  |  |  | Limit T(K) |  |  |
| Winding temperature |  |  |  |  | t1(°C) |  |  | R1(Ω) |  | t2(°C) |  |  |  |  | R2(Ω) |  |  | Temperature (°C) |  |  |  |  |  |
| ☐ impedance protected ☐ protected by protective device: Tf of motor protector(°C): operating time of motor protector(min) . |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test repeated operating time of motor protector(min) . |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test voltage(V) |  |  |  |  |  |  | Current(A) |  |  |  |  |  |  |  |  | Power input(W) |  |  |  |  |  |  |  |
| No. | Position |  |  |  |  |  |  |  | t (C) |  |  |  |  | T(K) |  |  |  |  |  |  | Limit T(K) |  |  |
| Winding temperature |  |  |  |  | t1(°C) |  |  | R1(Ω) |  | t2(°C) |  |  |  |  | R2(Ω) |  |  | Temperature (°C) |  |  |  |  |  |
| Check: ☐ The appliance emitted flames, molten metal, poisonous or ignitable gas in hazardous amounts ☐ Enclosures had deformed to such an extent that compliance with cl. 8 is impaired ☐ Appliance can still be operated and it didn’t comply with 20.2 ☐ the appliance shall not undergo a dangerous malfunction ☐ there shall be no failure of protective electronic circuits if the appliance is still operable ☐ The temprature of the housing of motor-compressors other than those which comply with IEC 60335-2-34 shall not exceed 150ºC. ☐ No above damage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ☐ 19.8 Three-phase motors operated at rated voltage with one phase disconnected for the period specified in 19.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test date: |  |  |  |  |  |  |  |  |  |  |  |  | Ambient: |  |  |  |  |  |  |  |  |  |  |
| Equipment ID: |  |  |  |  |  |  |  |  |  |  |  |  | Sample ID |  |  |  |  |  |  |  |  |  |  |
| Test voltage(V) |  |  |  |  |  |  | Current(A) |  |  |  |  |  |  |  |  | Power input(W) |  |  |  |  |  |  |  |
| No. |  |  |  | Position |  |  |  |  |  |  |  |  |  |  |  |  | t (C) |  |  | T(K) |  |  | Limit T(K) |
| Check: ☐ The appliance emitted flames, molten metal, poisonous or ignitable gas in hazardous amounts ☐ Enclosures had deformed to such an extent that compliance with cl. 8 is impaired ☐ Appliance can still be operated and it didn’t comply with 20.2 ☐ the appliance shall not undergo a dangerous malfunction ☐ there shall be no failure of protective electronic circuits if the appliance is still operable ☐ The temprature of the housing of motor-compressors other than those which comply with IEC 60335-2-34 shall not exceed 150ºC. ☐ No above damage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ☐ 19.10 Appliance incorporating series motors are operated with lowest possible load and supplied at 1,3 times rated voltage for 1min. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test date: |  |  |  |  | Ambient: |  |  |  |
| Equipment ID: |  |  |  |  | Sample ID |  |  |  |
| Test voltage(V) |  |  | Current(A) |  |  | Power input(W) |  |  |
| Check: ☐ parts were ejected from the appliance. ☐ The appliance emitted flames, molten metal, poisonous or ignitable gas in hazardous amounts ☐ Enclosures had deformed to such an extent that compliance with cl. 8 is impaired ☐ Appliance can still be operated and it didn’t comply with 20.2 ☐ the appliance shall not undergo a dangerous malfunction ☐ there shall be no failure of protective electronic circuits if the appliance is still operable ☐ The temprature of the housing of motor-compressors other than those which comply with IEC 60335-2-34 shall not exceed 150ºC. ☐ No above damage |  |  |  |  |  |  |  |  |

| ☐ 19.11 Abnormal operation of electronic circuits The appliance is test under rated voltage. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test date: |  |  |  | Ambient: |  |  |
| Equipment ID: |  |  |  | Sample ID |  |  |
| Description of fault conditions | Test time(min) |  | Test current(A) |  | Test results |  |
| Check: ☐ The appliance emitted flames, molten metal, poisonous or ignitable gas in hazardous amounts ☐ Enclosures had deformed to such an extent that compliance with cl. 8 is impaired ☐ Appliance can still be operated and it didn’t comply with 20.2 ☐ the appliance shall not undergo a dangerous malfunction ☐ there shall be no failure of protective electronic circuits if the appliance is still operable ☐ The temprature of the housing of motor-compressors other than those which comply with IEC 60335-2-34 shall not exceed 150ºC. ☐ No above damage |  |  |  |  |  |  |

| ☐ 19.11.3 Abnormal operation of protective electronic circuit If the appliance incorporates a protective electronic circuit which operates to ensure compliance with clause 19, the relevant test is repeated with a single fault simulated, as indicated in a) to f) of 19.11.2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test date: |  |  | Ambient: |  |  |
| Equipment ID: |  |  | Sample ID |  |  |
| Repeated clause |  | Failure of component in protective electronic circui |  | Result |  |
|  |  |  |  | ☐ Pass ☐ Fail |  |
|  |  |  |  | ☐ Pass ☐ Fail |  |
|  |  |  |  | ☐ Pass ☐ Fail |  |

☐ 19.11.4.8 Appliance incorporating an electronic circuit that relies on a programmable component to function correctly The appliance is supplied at rated voltage (AC: V) and operated under normal operation. After approximately 60 s, the power supply voltage is reduced to a level such that the appliance ceases to respond to user inputs or parts controlled by the programmable component cease to operate, whichever occurs first. This value of supply voltage is recorded: V. The appliance is supplied at rated voltage and operated under normal operation. The voltage is then reduced to a value of approximately 10 % less than the recorded voltage. It is held at this value for approximately 60 s and then increased to rated voltage. The rate of decrease and increase of the power supply voltage is to be approximately 10 V/s.

☐ The appliance shall continue to either operate normally from the same point in its operating cycle at which the voltage decrease occurred ☐ or a manual operation shall be required to restart it

| ☐ 19.12 Abnormal operation of electronic circuits |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test date: |  |  |  | Ambient: |  |  |
| Equipment ID: |  |  |  | Sample ID |  |  |
| If the safety of the appliance for any of the fault conditions specified in 19.11.2 depends on the operation of a miniature fuse-link complying with IEC 60127, the test is repeated but with the fuse replaced by an ammeter |  |  |  |  |  |  |
| Description of fault conditions | Rated current of fuse-link(A) |  | Measured current(A) |  | Test results |  |
| Check: ☐ The appliance emitted flames, molten metal, poisonous or ignitable gas in hazardous amounts ☐ Enclosures had deformed to such an extent that compliance with cl. 8 is impaired ☐ Appliance can still be operated and it didn’t comply with 20.2 ☐ the appliance shall not undergo a dangerous malfunction ☐ there shall be no failure of protective electronic circuits if the appliance is still operable ☐ The temprature of the housing of motor-compressors other than those which comply with IEC 60335-2-34 shall not exceed 150ºC. ☐ No above damage |  |  |  |  |  |  |

| ☐ 19.101 abnormal test of heating systems |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test date: |  |  |  |  |  | Ambient: |  |  |  |  |  |  |
| Equipment ID: |  |  |  |  |  | Sample ID |  |  |  |  |  |  |
| Heating systems are continuously energized at a voltage equal to 1.1 times their working voltage, until steady conditions are established. |  |  |  |  |  |  |  |  |  |  |  |  |
| Test voltage(V) |  |  |  | Current(A) |  |  | Power input(W) |  |  |  |  |  |
| No. | Position |  |  |  |  |  |  | t (C) |  | T(K) |  | Limit T(K) |
| Check: ☐ The appliance emitted flames, molten metal, poisonous or ignitable gas in hazardous amounts ☐ Enclosures had deformed to such an extent that compliance with cl. 8 is impaired ☐ Appliance can still be operated and it didn’t comply with 20.2 ☐ the appliance shall not undergo a dangerous malfunction ☐ there shall be no failure of protective electronic circuits if the appliance is still operable ☐ The temprature of the housing of motor-compressors other than those which comply with IEC 60335-2-34 shall not exceed 150ºC. ☐ No above damage |  |  |  |  |  |  |  |  |  |  |  |  |

| ☐ 19.102 abnormal test of ice-marker and ice-cream appliance Apply any defect which may be expected in normal use. |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test date: |  |  |  |  |  |  | Ambient: |  |  |  |  |
| Equipment ID: |  |  |  |  |  |  | Sample ID |  |  |  |  |
| Test voltage(V) |  |  |  | Current(A) |  |  |  | Power input(W) |  |  |  |
| Fault condition: |  |  | Position |  | Measured temperature |  |  |  | remark |  |  |
| Timer stop |  |  |  |  |  |  |  |  |  |  |  |
| Disconnection and reconnection the supply |  |  |  |  |  |  |  |  |  |  |  |
| Failure of valve |  |  |  |  |  |  |  |  |  |  |  |
| Open-circuit or short-circuit of components |  |  |  |  |  |  |  |  |  |  |  |
| Check: ☐ The appliance emitted flames, molten metal, poisonous or ignitable gas in hazardous amounts ☐ Enclosures had deformed to such an extent that compliance with cl. 8 is impaired ☐ Appliance can still be operated and it didn’t comply with 20.2 ☐ the appliance shall not undergo a dangerous malfunction ☐ there shall be no failure of protective electronic circuits if the appliance is still operable ☐ The temperatures of the windings shall not exceed the values given in table 8 ☐ The temprature of the housing of motor-compressors other than those which comply with IEC 60335-2-34 shall not exceed 150ºC. ☐ No above damage |  |  |  |  |  |  |  |  |  |  |  |

| ☐ 19.103 abnormal test of appliance intend for camping use |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test date: |  |  |  |  |  | Ambient: |  |  |  |  |  |
| Equipment ID: |  |  |  |  |  | Sample ID |  |  |  |  |  |
| The appliance is placed on a support inclined by 5º in the most unfavourable position and is operated under normal operation at rated voltage until steady conditions are established. |  |  |  |  |  |  |  |  |  |  |  |
| Test voltage(V) |  |  |  | Current(A) |  |  | Power input(W) |  |  |  |  |
| No. | Position |  |  |  |  |  |  |  | T(K) |  | Limit T(K) |
| Check: ☐ The appliance emitted flames, molten metal, poisonous or ignitable gas in hazardous amounts ☐ Enclosures had deformed to such an extent that compliance with cl. 8 is impaired ☐ Appliance can still be operated and it didn’t comply with 20.2 ☐ the appliance shall not undergo a dangerous malfunction ☐ there shall be no failure of protective electronic circuits if the appliance is still operable ☐ During the test, non-self-resetting thermal cut-outs shall not operate and no ignitable gas shall accumulated in appliance. ☐ The temprature of the housing of motor-compressors other than those which comply with IEC 60335-2-34 shall not exceed 150ºC. ☐ No above damage |  |  |  |  |  |  |  |  |  |  |  |

| ☐ 19.104 abnormal test of illuminating equipment |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test date: |  |  |  |  |  | Ambient: |  |  |  |  |  |
| Equipment ID: |  |  |  |  |  | Sample ID: |  |  |  |  |  |
| The appliance fitted with a lamp and with a rated wattage equal to the maximum rated wattage marked on the appliance, is operated for 12h at 1.06 times rated voltage. If the lamp does not attain the maximum rated wattage at rated voltage, the voltage is varied until the maximum rated wattage is reached and then increased to 1.06 times this voltage. |  |  |  |  |  |  |  |  |  |  |  |
| Test voltage(V) |  |  |  | Current(A) |  |  | Power input(W) |  |  |  |  |
| No. | Position |  |  |  |  |  |  |  | T(K) |  | Limit T(K) |
| Check: ☐ The appliance emitted flames, molten metal, poisonous or ignitable gas in hazardous amounts ☐ Enclosures had deformed to such an extent that compliance with cl. 8 is impaired ☐ Appliance can still be operated and it didn’t comply with 20.2 ☐ the appliance shall not undergo a dangerous malfunction ☐ there shall be no failure of protective electronic circuits if the appliance is still operable ☐ The temprature of the housing of motor-compressors other than those which comply with IEC 60335-2-34 shall not exceed 150ºC. ☐ No above damage |  |  |  |  |  |  |  |  |  |  |  |

| ☐ 19.105 abnormal test |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test date: |  |  |  |  |  | Ambient: |  |  |  |  |  |
| Equipment ID: |  |  |  |  |  | Sample ID: |  |  |  |  |  |
| The appliance is operated under the conditions specified in Clause 11 but with a fully charged 70Ah battery connected withreversed polarity. |  |  |  |  |  |  |  |  |  |  |  |
| Test voltage(V) |  |  |  | Current(A) |  |  | Power input(W) |  |  |  |  |
| No. | Position |  |  |  |  |  |  | T(K) |  |  | Limit T(K) |
| Check: ☐ The appliance emitted flames, molten metal, poisonous or ignitable gas in hazardous amounts ☐ Enclosures had deformed to such an extent that compliance with cl. 8 is impaired ☐ Appliance can still be operated and it didn’t comply with 20.2 ☐ the appliance shall not undergo a dangerous malfunction ☐ there shall be no failure of protective electronic circuits if the appliance is still operable ☐ The temprature of the housing of motor-compressors other than those which comply with IEC 60335-2-34 shall not exceed 150ºC. ☐ No above damage |  |  |  |  |  |  |  |  |  |  |  |

| ☐ 19.13 Electric strength tests after abnormal operation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test date: |  |  | Ambient: |  |  |
| Equipment ID: |  |  | Sample ID: |  |  |
| The tests are carried out after the appliance has cooled down to room temperature. |  |  |  |  |  |
| Test voltage applied between: |  | Voltage (V) |  | Breakdown (Yes/No) |  |
| ☐ after the operation or interruption of a control, clearance and creepage distances across the functional insulation shall withstand the electric strength of 16.3, the test voltage=2xUn=______V |  |  |  |  |  |
| Notes: For appliances which are immersed in or filled with conducting liquid in normal use, the appliance is immersed in or filled with water for 24h before the electric strength test is carried out. |  |  |  |  |  |

| ☐ 19.14 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test date: |  |  |  |  |  |  |  | Ambient: |  |  |  |  |  |
| Equipment ID: |  |  |  |  |  |  |  | Sample ID: |  |  |  |  |  |
| Appliances are tested under the conditions of clause11. any contactor or relay contact that operates under the conditions of clause 11 short-circuited. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test voltage(V) |  |  |  |  | Current(A) |  |  |  |  | Power input(W) |  |  |  |
| No. | Position |  |  |  |  | t (C) |  |  | T(K) |  |  | Limit T(K) |  |
| Check: ☐ self-resetting thermal cut-out ☐ non-self-resetting thermal cut-out ☐ thermal link ☐ No thermal cut-out operated |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Operating temperature of thermal cut out(°C) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ☐ The appliance emitted flames, molten metal, poisonous or ignitable gas in hazardous amounts ☐ Enclosures had deformed to such an extent that compliance with cl. 8 is impaired ☐ Appliance can still be operated and it didn’t comply with 20.2 ☐ the appliance shall not undergo a dangerous malfunction ☐ there shall be no failure of protective electronic circuits if the appliance is still operable ☐ No above damage |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ☐ 20 Stability test |  |  |  |
| --- | --- | --- | --- |
| Test date: |  | Ambient: |  |
| Equipment ID: |  | Sample ID: |  |
| ☐ 20.102 Stability test ☐ Appliace provided with doors shall be loaded using cylindrical weights having a diameter of 80mm and a mass of 0.5kg. ☐ Pass ☐ Fail ☐ 20.103 Stability test ☐ Appliace provided with sliding drawers inside food storage compartments shall be loaded with a uniformly distributed load/unit storage volume of drawer of 0.5kg/l ☐ Pass ☐ Fail ☐ 20.104 Stability test ☐ Appliace provided with sliding drawers accessible without opening a door shall be loaded with a uniformly distributed load/unit storage volume of drawer of 0.5kg/l ☐ Pass ☐ Fail |  |  |  |

| ☐ 21.1 Mechanical strength |  |  |  |
| --- | --- | --- | --- |
| Test date: |  | Ambient: |  |
| Equipment ID: |  | Sample ID: |  |
| The appliance is rigidly supported and three blows, having an impact energy of 0,5 J, are applied to every point of the enclosure that is likely to be weak Test part: Result ☐ Pass ☐ Fail |  |  |  |

| ☐ 21.2 strength test of accessible parts of solid insulation |  |  |  |
| --- | --- | --- | --- |
| Test date: |  | Ambient: |  |
| Equipment ID: |  | Sample ID: |  |
| Thickness of supplementary insulation: Thickness of reinforced insulation: 0 testing as follows: The insulation is raised to the temperature measured during the test of clause 11. the surface of the insulation is then scratched by means of a hardened steel pin, the pin is held at an angle of 80℃ -85℃ to the horzontal and loaded so that the force exered along its axis is 10N± 0.5N. the scatches are made by drawing the pin along the surface of the insulation at a speed of approximately 20mm/s. Two parallel scratches are made. They are spaced surfficiently apart so that they are not affected by each other, their length covering approximaately 25% of the length of insulation. Two similar scratches are made at 90 º to the first pair without crossing them. The test fingernail of figure 7 is then applied to the scrateched surface with a force of appr. 10N. No further damage, such as separation of the material, shall occur. The insulation shall then withstand the electric strenth test of 16.3 The hardend steel pin is then applied perpendicularly with a force of 30N ± 0.5N to an unscratched part of the surface. The insulation shall then withstand the electric strength test of 16.3 with the pin still applied and used as one of the electrodes. Test part/test voltage Result ☐ Pass ☐ Fail ☐ Pass ☐ Fail ☐ Pass ☐ Fail ☐ Pass ☐ Fail |  |  |  |

| ☐ 21.101 Mechanical strength |  |  |  |
| --- | --- | --- | --- |
| Test date: |  | Ambient: |  |
| Equipment ID: |  | Sample ID: |  |
| Test condition: ☐ The appliance is placed on a horizontal wooden panel which is dropped 50 times from a height of 50 mm onto a solid base of wood. ☐ Pass ☐ Fail ☐ The appliance is then fastened in its normal position of use to a vibration-generator by means of straps around the enclosure. The type of vibration is sinusoidal, the direction is vertical and the severity is as follows: duration: 30min; amplitude: 0.35mm; sweep frequency range: 10Hz, 55Hz, 10Hz; sweep rate: approximately one octave per minute. ☐ Pass ☐ Fail |  |  |  |

| ☐ 22.5 Discharge of capacitor |  |  |  |
| --- | --- | --- | --- |
| Test date: |  | Ambient: |  |
| Equipment ID: |  | Sample ID |  |
| The appliance is supplied at rated voltage, any switch is then placed in the off-position and the appliance is disconnected from the supply mains. One second after disconnection, the voltage between the pins of plug is measured. If compliance relies on the operation of an electronic circuit, the electromagnetic phenomena tests of 19.11.4.3 and 19.11.4.4 are applied one at a time to the appliance. The discharge test is then repeated three times and for each test Switch in OFF position, disconnected from the supply mains at the instant of voltage peak, measure the voltage of pins of plug after one second; the voltage shall not exceed 34V. repeat with plug reversed when possible Normal order Test result: V Reverse plug Test result: V |  |  |  |

| ☐ 22.7 Pressure test |  |  |  |
| --- | --- | --- | --- |
| Test date: |  | Ambient: |  |
| Equipment ID: |  | Sample ID |  |
| Compression-type appliance, including protective enclosure of a protected cooling system, using flammable refrigerants shall withstand - a pressure of 3.5 times the saturated vapour pressure of the refrigerant at 70ºC for parts exposed to the high-side pressure during normal operation; - a pressure of 5 times the saturated vapour pressure of the refrigerant at 20ºC for parts exposed to the low-side pressure during normal operation; The pressure is maintained for 1 min. ☐ Pass ☐ Fail |  |  |  |

| ☐ 22.47 static pressure test for appliance connected to the water mains |  |  |  |
| --- | --- | --- | --- |
| Test date: |  | Ambient: |  |
| Equipment ID: |  | Sample ID |  |
| Test pressure: ___________MPa (twice the maximum inlet water pressure of 1,2MPa), 5min There shall be no leakage from any part, including any inlet water hose. ☐ Pass ☐ Fail |  |  |  |

| ☐ 22.102 |  |  |  |
| --- | --- | --- | --- |
| Test date: |  | Ambient: |  |
| Equipment ID: |  | Sample ID |  |
| Insulated wire heaters shall be protected against entry of water. Immersing three samples of the complete heating element in water containing approximately 1% NaCl and having a temperature of 20ºC±5ºC for a period of 24h. A voltage of 1250V is then applied for 15 min between the live part of the heating element and the water. ☐ Pass ☐ Fail |  |  |  |

| ☐ 22.103 |  |  |  |
| --- | --- | --- | --- |
| Test date: |  | Ambient: |  |
| Equipment ID: |  | Sample ID |  |
| Appliances employing a transcritical refrigeration system shall in the high pressure side of the refrigeration system include a pressure relief device on the compressor or between the compressor and the gas cooler. There shall be no shut off devices or other components except piping between the compressor and the pressure relief device, which could introduce a pressure drop. Compliance is checked by inspection and by the tests specified.The pressure is maintained for one minute and the parts under test shall show no leakage. ☐ Pass ☐ Fail |  |  |  |

| ☐ 22.107 anti-explosive test |  |  |  |
| --- | --- | --- | --- |
| Test date: |  | Ambient: |  |
| Equipment ID: |  | Sample ID |  |
| 22.107.1 For appliance with a protected cooling system and flammable refrigerants used. Simulated the leakage of refrigerant at the critical point. The quantity of refrigerant equal to 80% of the nominal charge of the refrigerant ±1.5g or the maximum that can be injected in one hour. Measure the concentration of leakage refrigerant from the beginning of the test and for at least 1h after injection of the gas stopped. ☐ Pass ☐ Fail 22.107.2 Scratched the accessible surfaces of protected cooling system: - force at right angles to the surface: 35N±3N - force parallel to the surface: not exceeding 250N after test, the appropriate part shall withstand the test of 22.7, the test pressure being reduced by 50%. ☐ Pass ☐ Fail |  |  |  |

| ☐ 22.108 anti-explosive test |  |  |  |
| --- | --- | --- | --- |
| Test date: |  | Ambient: |  |
| Equipment ID: |  | Sample ID |  |
| For appliance with a protected cooling system and flammable refrigerants used. Simulated the leakage of refrigerant at the center of the back wall of the compartment at a distance from the top of the compartment approximately equal to one-third of the height of the compartment. The quantity of refrigerant equal to 80% of the nominal charge of the refrigerant ±1.5g. Measure the concentration of leakage refrigerant from the beginning of the test and for a period of 15min after a sustained decrease is observed ☐ Pass ☐ Fail |  |  |  |

| ☐ 22.109 anti-explosive test |  |  |  |
| --- | --- | --- | --- |
| Test date: |  | Ambient: |  |
| Equipment ID: |  | Sample ID |  |
| For compression-type appliances which use flammable refrigerant. A quantity of refrigerant equal to 50% of the nominal charge of the refrigerant ±1.5g is injected into the considered area. Injection is to be at constant rate over a period of 1h and is to be at the point of closest approach of - pipework joints in external parts of the cooling circuit, or - the gasket of semi-hermetic motor-compressors Measure the concentration of leakage refrigerant from the beginning of the test until 15min after a sustained decrease is observed ☐ Pass ☐ Fail |  |  |  |

| ☐ 22.116 |  |  |  |
| --- | --- | --- | --- |
| Test date: |  | Ambient: |  |
| Equipment ID: |  | Sample ID |  |
| Accessible glass panels with an area having any two orthogonal dimensions exceeding 75 mm shall be either made from glass that shatters into small pieces when broken or be made from glass that has enhanced mechanical strength Test is performed as specified. At the conclusion of the tests the glass shall not be broken or cracked. ☐ Pass ☐ Fail |  |  |  |

| ☐ 23.3 Flexing test for internal wire |  |  |  |
| --- | --- | --- | --- |
| Test date: |  | Ambient: |  |
| Equipment ID: |  | Sample ID |  |
| The appliance is supplied at rated voltage and operated under normal operation. The movable part is moved backwards and forwards, so that the conductor is flexed through the largest angle allowed by the construction, the rates is 30 per minute, the number or flexing is ☐ 10000, for conductors flexed during normal use ☐ 100, for conductors flexed during user maintenance; ☐ Instead of the test being carried out while the appliance is in operation, it is carried out with the appliance disconnected from the supply. The number of flexings for conductors flexed during normal use is increased to 100 000. After this test, a test voltage of 1000V being applied between live parts and accessible metal parts. ☐ Pass ☐ Fail Record: The type of interal wire: . Check: ☐ The compliance with this standards is impaired: . ☐ The appliance wasn’t fit for further use ☐ No above damage |  |  |  |

| ☐ 23.5 electrical stress test for internal wire |  |  |  |
| --- | --- | --- | --- |
| Test date: |  | Ambient: |  |
| Equipment ID: |  | Sample ID |  |
| The basic insulation shall be electrically equivalent to the basic insulation of cord complying with IEC 60227 or IEC 60245 or comply with the following electric strength test A voltage of 2000V is applied for 15min between the conductor and metal foil wrapped around the insulation. There shall be no breakdown. ☐ Pass ☐ Fail Record: The type of interal wire: . |  |  |  |

| ☐ 24.5 capacitors in auxiliary windings |  |  |  |
| --- | --- | --- | --- |
| Test date: |  | Ambient: |  |
| Equipment ID: |  | Sample ID |  |
| Test voltage: 1.1Un=__________V Test load: minimum load Voltage across the capacitor measured: __________V Rated voltage of the capacitor:___________V ☐ Pass ☐ Fail Remark: for starting capacitors, the voltage across the capacitors shall not exceed 1,3 times the rated voltage of the capacitor when the appliance is operating at 1,1 timers the rated voltage. |  |  |  |

| ☐ 25.14 Flexing test for supply cord |  |  |  |
| --- | --- | --- | --- |
| Test date: |  | Ambient: |  |
| Equipment ID: |  | Sample ID |  |
| The conductors are supplied at rated voltage and loaded with rated current of the appliance. The oscillating member is moved through an angle of 90° ,the rates is 60 per minute, the number or flexing is ☐ 10000, for type X or type Y attachment; ☐ 20000, for type Z attachment; The cord is loaded a force is ☐ 10N, for cords having a nominal cross-sectional area exceeding 0,75mm2; ☐ 5N, for other cords; ☐ The cord and its associated parts are turned through an angle of 90° after half of the number of flexings; ☐ a flat cord is fitted; Record: The type of supply cord: . The total number of strands: ;the breakage number of strands: . At the end of test; the current is: . Check: ☐ A short circuit between the conductors; ☐ a breakage of more than 10% of the strands of any conductors ☐ separation of the donductor from its terminal ☐ loosening of any cord guard; ☐ damage to the cord or cord guard which could impair compliance with this strandard: . ☐ broken strands piercing the insulation and becoming accessible ☐ No above damage |  |  |  |

| ☐ 25.15 pull and torque test of supply Cord |  |  |  |
| --- | --- | --- | --- |
| Test date: |  | Ambient: |  |
| Equipment ID: |  | Sample ID |  |
| Mass of Appliance: __________kg Pull force: ________N Torque: _______Nm The cord is pulled, without jerking, for 1s in the most unfavourable direction with the force specified. The test is carried out 25 times. The cord, unless on the automatic cord reel, is then subjected to a torque that is applied as close as possible to the appliance. Applied for 1min During the test, the cord shall not be damaged, and shall show no appreciated strain at the terminals. The cord shall not be displaced by more than 2mm Remark: displacement of cord: ______mm ☐ Pass ☐ Fail |  |  |  |

| ☐ 27.5 Earthing resistance test |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test date: |  |  | Ambient: |  |  |  |
| Equipment ID: |  |  | Sample ID |  |  |  |
| The earthing resistance shall not exceed 0,1 Ω. Note: The resistance of supply cord is not included. Test current: ☐ 25A ☐ 1,5 times rated current of the appliance: A Record: |  |  |  |  |  |  |
| No. | Test location |  |  | Earthing resistance (Ω) |  | Result |
| 1 | Earthing terminal(contact)- |  |  |  |  | ☐ Pass ☐ Fail |
|  |  |  |  |  |  | ☐ Pass ☐ Fail |

| ☐ 28.1 Torque test for screws and nuts |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test date: |  |  |  |  | Ambient: |  |  |  |
| Equipment ID: |  |  |  |  | Sample ID |  |  |  |
| Screws and nuts are tested if they are -used for electric connections; -used for connections providing earthing continuity, unless at least two screws or nuts used. -like to be tightened during user maintenance or installation or replacing supply cord of type X attachment ☐ 10 times for screws engagement with a thread of insulating material ☐ 5 times for nuts and other screws |  |  |  |  |  |  |  |  |
| Threaded part identification | Diameter of thread (mm) |  | Column number (I, II, or III) | Applied torque ( Nm ) |  | Test times |  | Result |
|  |  |  |  |  |  |  |  | ☐ Pass ☐ Fail |
|  |  |  |  |  |  |  |  | ☐ Pass ☐ Fail |

| ☐ 30.1 Ball-pressure test |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test date |  |  |  |  | Test Ambient |  |  |
| Equipment ID: |  |  |  |  | Sample ID |  |  |
| Test requirements | External parts of non-metallic material, parts of insulating material supporting live parts, including connections and parts of thermoplastic material providing supplementary insulation or reinforced insulation, the deterioration of which might cause the tool fail to comply with this standard, shall be sufficiently resistant to heat. |  |  |  |  |  |  |
| Parts tested | Material |  |  | Cabinet temp. (oC) – 1h [required: max. Value of ΔT+40, 75 for external part; 125 for retain live part] |  |  | Diameter of impression (mm) [required ≤2] |
| Conclusion 结论 |  | ☐Pass ☐ Fail |  |  |  |  |  |

| ☐ 30.2 Glow-wire test |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test date |  |  |  |  | Test Ambient |  |  |  |  |  |  |
| Equipment ID: |  |  |  |  | Sample ID |  |  |  |  |  |  |
| Test requirements 试验要求 |  |  | Glow-wire test is made at a temperature of 850℃ on parts of non-metallic material. The duration (te) from the beginning of the tip application up to the time when flames extinguish should be no more than that the duration of glow-wire application (ta) plus 30s. 对于非金属材料部件，应经受850℃的灼热丝试验。样品的的任何火焰在移开灼热丝30S后熄灭 |  |  |  |  |  |  |  |  |
| Parts tested | Temp. |  |  | Flaming and glowing occurred, ignition of wrapping tissue (Yes/No) 燃烧和炽热发生, 薄纸燃烧 |  | Flame height 焰高(mm) | ti 起燃 (s) | te 熄灭 (s) |  | ta 接触 (s) | Required value 要求值 (s) |
|  | 850 |  |  |  |  |  |  |  |  |  | te< ta+30 |
|  | 850 |  |  |  |  |  |  |  |  |  | te< ta+30 |
|  | 850 |  |  |  |  |  |  |  |  |  | te< ta+30 |
|  | 850 |  |  |  |  |  |  |  |  |  | te< ta+30 |
|  | 850 |  |  |  |  |  |  |  |  |  | te< ta+30 |
| Conclusion 结论 |  | ☐Pass ☐ Fail |  |  |  |  |  |  |  |  |  |

| ☐Annex E TABLE: Needle- flame test (NFT) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test date |  |  |  | Test Ambient |  |  |
| Equipment ID: |  |  |  | Sample ID |  |  |
| Object/ Part No./ Material 部件 |  |  | Duration of application of test flame (ta); (s) 测试接触 时间 | Ignition of specified layer Yes/No 是否单层燃烧 | Duration of burning (tb) (s) 燃烧时间 |  |
| Remark |  |  | The duration of burning (tb) shall not exceed 30 s. However, for printed circuit boards, the duration of burning shall not exceed 15 s. |  |  |  |
| Conclusion 结论 | ☐Pass ☐ Fail |  |  |  |  |  |

| ☐ Annex N & 29.2 Proof tracking index test |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test date |  |  |  |  | Test Ambient |  |  |
| Equipment ID: |  |  |  |  | Sample ID |  |  |
| Test requirements |  | If CTI of material is unknown, PTI tested in accordance with IEC 60112 |  |  |  |  |  |
| Tested on |  |  |  | Proof voltage (V) |  | Flashover or breakdown (Yes/No) |  |
| Conclusion 结论 | ☐Pass ☐ Fail |  |  |  |  |  |  |

| ☐ Annex T UV-C radiation effect on non-metallic materials |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test date: | See Below |  |  | Ambient: |  |  |  |
| Equipment ID: |  |  |  | Sample ID |  |  |  |
| Testing condition: |  |  |  |  |  |  |  |
| Oven Temperature: |  | 63°C ±3°C |  |  |  |  |  |
| UV-C spectral irradiance: |  | 10W/ m2 @ 254nm |  |  |  |  |  |
| UV-C exposure hours: |  | 1000 hours |  |  |  |  |  |
| Sample name: | UV expose period: |  | Chapy impact retention rate(%) |  |  | Tensile strenghth retention rate(%) | Result |
| ① |  |  |  |  |  |  | ☐Pass ☐ Fail |
| ② |  |  |  |  |  |  | ☐Pass ☐ Fail |

| ☐Annex EE TABLE: Test for material encasing and in contact with thermal insulation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test date |  |  | Test Ambient |  |
| Equipment ID: |  |  | Sample ID |  |
| The flame is applied for 5 s ± 0,5 s and then removed for 5 s ± 0,5 s. The application cycle is repeated until the test specimen has been subjected to five applications of the test flame. Following each flame application, the laboratory burner is immediately withdrawn to a distance not less than 150 mm so that is has no effect on the test specimen. During and after the test, there shall be no burn-through and no flaming particles or drops which ignite the cotton pad indicator. |  |  |  |  |
| Conclusion 结论 | ☐Pass ☐ Fail |  |  |  |

End